UNITED STATES PATENT OFFICE.

___________________________________

FRANCIS ALGER, OF BOSTON, MASSACHUSETTS.

___________________________________

IMPROVEMENT IN REMOVABLE CHARGE-CHAMBERS FOR EXPLOSIVE SHELLS.

___________________________________

Specification forming part of Letters Patent No. 39,267, dated July 21, 1863.

___________________________________

To all whom it may concern:

Be it known that I, FRANCIS ALGER, of the city of Boston, in the county of Suffolk and the State of Massachusetts, have invented a new and improved method of lodging and protecting a charge of gunpowder within an explosive projectile, the nature of which I hereby proceed to describe, reference being made to the accompanying drawings.

The invention is more particularly intended for the Shrapnel or case shot of rifled cannon, but is also applicable to spherical case-shot, and to other projectiles.

It consists in a water-proof tube or pouch attached to the fuse, in which tube sufficient powder is contained to burst the projectile, and which, aft4r having been filled and attached to the fuse, is inserted into the shell through its fuse-hole.

The usefulness of the invention consists in this, that the bursting-charge may at any time be removed from the projectile by simply withdrawing the fuse with its attached pouch, and may be replaced again with equal facility.

In the accompanying drawings, A, Figure 1, represents the pouch or tube for containing the powder.  This I make of india-rubber cloth, of leather, or of other suitable material, care being taken not to bring a surface of india-rubber into direct contact with gunpowder.

Fig. 2 represents a fuse (in this case the Schenkl combination-fuse) prepared for the attachment of the tube A, for which purpose a groove, c, has been turned on it.  By means of this groove the pouch A, after having been filled, is made fast to the fuse, as shown in Fig. 3.  The fuse and pouch are then ready for insertion into the shell.

I am aware that tubes or pouches of metal for containing the bursting-charges of Shrapnel and of incendiary shells have been known and employed, and that such tubes have been attached permanently to the fuse, and I do not claim the general use of such; but

What I claim is as follows:

The use of a tube or pouch of india-rubber cloth, or of other flexible material, containing gunpowder and attached to a fuse, so as to form, together with the fuse, one apparatus, which may be inserted into or removed from a projectile at a single operation.

FRANCIS ALGER.

Witnesses:

B. F. CRAIG,

W. W. JACKSON.