UNITED STATES PATENT OFFICE.

______________________________________

HALVOR HALVORSON, OF CAMBRIDGE,

ASSIGNOR TO CHARLES SPEAR, OF BOSTON, MASSACHUSETTS.

______________________________________

IMPROVEMENT IN EXPLOSIVE SHELLS.

______________________________________

Specification forming part of Letters Patent No. 40,538, dated November 3, 1863.

______________________________________

To all whom it may concern:

Be it known that I, HALVOR HALVORSON, of Cambridge, in the county of Middlesex and State of Massachusetts, have invented a certain new and useful Improvement on Shells for Ordnance; and I do hereby declare that the following is a full, clear, and exact description thereof, reference being had to the accompanying drawings, and to the letters and marks thereon.

By the drawings forming part of this specification a shell is represented having my improvement as a part thereof, Figure 1 of these drawings being a view by elevation; Fig. 2, a view by vertical longitudinal section, and Fig. 3 being a view of the base or bottom of the shell without the base-plate in position.  In each of these figures, where like parts are shown, like letters and marks are used to indicate the parts.

The shell here represented is of cylindrical form, having a mouth, a, in communication with a central passage, b, at the lower sides of which are openings c to channels d, that terminate on the periphery e of the shell.  A base-plate, b, with suitable packing properly secured, will close the lower end of the central passage, f, so that when the shell has been discharged from the gun or mortar the air passing through the mouth, central passage, openings, and channels will produce rotation of the shell.

The space g is the body of the shell for the explosive material may be as much greater or less than that here shown as may be deemed necessary for the particular kind of shell or for the peculiar character of the explosive material.

A recessed annular cap, h, the sides of which are indicated in Fig. 2 by blue lines, covers and embraces the sides of the opening to the explosion-chamber of the shell.  (Indicated by red lines.)  This cap may be made of any suitable metal or other material, holding a fulminate on the interior of the cap, and so affixed or attached to the shell as may be deemed best for the purpose required.  It is evident that this shell being properly loaded and capped, cannot fall or strike without exploding, and therefore establishing its superiority over those whose fulminate is at the apex of a conical form and much liable to glance and not explode.

What I claim as my invention, and desire to secure by Letters Patent, is –

1. The exterior explosive chamber lying exterior and around the central passage, and in combination therewith and with the month, and air openings and passages for producing rotation of the shell, as herein set forth.

2. The cap h, in combination with the explosive chamber g, as and for the purposes herein described.

This specification signed this 9th day of July, 1863.

HALVOR HALVORSON.

Witnesses:

DAVID W. CHILD.

TYLER R. WASGATT, Jr.